# Cisco IP Phone 7821

# Navigating

## Line Buttons

Two line buttons are located on the left of the display.

## Softkeys

Softkeys are feature buttons that dynamically change to reflect your current options. Four softkey buttons are located below the phone screen. You can press the More softkey () when available to reveal additional options.

## Scrolling

To scroll though a menu or list, press the up or down arrows on the Navigation Pad.

A scroll bar on the screen indicates your relative position within a list. Press the middle of the Navigation Pad to make your selection.

# Dialing

To place a call, pick up the handset and enter a number. Or, try one of these alternatives.

## Redial the last number

Press the Redial softkey to redial on your primary line. To redial on a secondary line, press the line button first.

## Dial on-hook

- Enter a number when the phone is idle.

- Lift the handset or press a line button, the Call softkey, the Headset button , the Speakerphone button, or the round Select button in the Navigation pad.

# Answer a Call

New call indicators:

- A flashing amber line button

- An animated icon and caller ID

- A flashing red light on your handset

To answer the call, lift the handset, press the flashing amber line button, the Answer softkey, or the Headset or Speakerphone button.

## Answer with multiple Lines

If you are talking on the phone when you get another call, a message appears briefly on the phone screen. Press the flashing amber line button to answer the second call and put the first call on hold automatically.

# Hold

Press the Hold  button . The hold icon appears and the line button flashes green.

To resume a call from hold, press the flashing green line button, Resume or Hold softkey.

# Transfer

- From a connected call (not on hold), press the Transfer  button.

- Enter the transfer recipient’s phone number.

- Announce the call if you wish after the recipient answers. Then press the Transfer button or the Transfer softkey. Or you can press Transfer before the recipient answers.

# Conference

- From a connected call (not on hold), press the Conference  button. This places the initial call on hold.

- Make a new call.

- Press the Conference button or the Conference softkey before or after the party answers. The conference begins and the phone displays “Conference.”

- Repeat these steps to add more participants.

The conference calls ends when last internal Brown person hangs up.

## View and Remove Participants

During a conference, press the Details softkey. To remove a participant from the conference, scroll to the participant and press the Remove softkey.

# Cisco IP Phone 7821

# Voicemail

When you have voicemail messages waiting, you will see two new message indicators:

- A solid red light on your handset

- New voicemail icon (O_O) on your display

## Listen to Messages

Press the Messages button  and follow the voice prompts. To check messages for a specific line, press the line button first.

If you forget your voicemail PIN, send an email to voicemail@brown.edu or call 32007.

## Divert to Voicemail

Press the Decline softkey when the call is ringing, active, or on hold. Divert sends an individual ringing call directly to your voicemail.

# Forward Calls

- To forward calls on your primary line to another number, press the Fwd All softkey.

- Enter an internal 5-digit phone number or press the Messages button to forward all calls to your voicemail.

- Look for confirmation of the forward on your phone display.

- To cancel call forwarding, press Fwd Off.

# Directories

You can use the Directories to search for and call any colleague by name.

- Press the Contacts button .

- Select a directory using the Navigation pad arrows to scroll through the list and the center Select button to choose a directory.

- Use your keypad to input search criteria. Use the backspace softkey to undo.

- Press the Search softkey.

- To dial, scroll to a listing using the navigation pad and press the Dial softkey.

# Call History (Recents)

To view your call history:

- Press the Applications  button.

- Press the center button of the Navigation pad to select Recents.

- Select a line to view. Your phone displays the last 150 missed, placed and received calls.

- To view details for a call, scroll to the call and press the More () softkey, then press Details.

- To dial from the call history, scroll to a listing then either lift the handset, press a line button, or press the Select button.

To edit a number before dialing, press the More softkey then EditDial.

## Speed Dial

To set up speed dial numbers, please send an email to telecom@brown.edu or call 32007.

CISCO 7821 IP Phone